User GUIDE

- Open simulator-1.0-SNAPSHOT.jar

- Click Button [IPL], and the Simulator is initialized.

If you want to use the simulator, you must click IPL first.

How to Test Floating Point Instructions/Vector Operations

Here are some instructions:

- Deposit FR0 and memory[22] a floating point number first. (e.g. FR0 = 2.0, memory[22] = 3.25)

- FADD FR0,0,0,22 (FR0 = FR0 + memory[22])

0110110000010110

- FSUB FR0,0,0,22 (FR0 = FR0 – memory[22])

0111000000010110

- Deposit R0 0 or 1, if R0 = 0, deposit memory[23] a floating point number, if R0 = 1, deposit memory[23] a fixed point number.

- CNVRT R0,0,0,23 (if R0 = 0, R0 = (float->int) memory[23]; if R0 = 1, FR0 = (int->float) memory[23])

0111110000010111

- Deposit some values to memory and registers

| FR0 | 2.0 |
| --- | --- |
| Memory[20] | 100 |
| Memory[21] | 200 |
| Memory[100] | 1 |
| Memory[101] | 2 |
| Memory[200] | 3 |
| Memory[201] | 4 |

- VADD FR0,0,0,20

(Vector1 starts from 100(memory[20]) and Vector2 starts from 200(memory[21]))

1000110000010100

- VSUB FR0,0,0,20

1001000000010100

- memory[24] = 3;

memory[25] = 0.25(0010000000000000->Fixed-point decimal representation);

- LDFR FR0,0,0,24 (FR0 = memory[24]. memory[25])

1010000000011000

- STFR FR0,0,0,24 (memory[24]. memory[25] = FR0)

1010010000011000

How to Run Program2 (included in Part3)

Program2 is pre-stored in a file, thus you can select Program2 in ComboBox [programNameSelector] and see pre-stored instructions for program 2 in Program Editor.

To test program 2:

- Click Button [PreStroeMemoryForProgram2]

That will store some values into memory, which helps run the program2;

| memory[12] | 0000001000000000 | 512 | Word store start |
| --- | --- | --- | --- |
| memory[13] | 0000000001000000 | 64 | for X1 |
| memory[14] | 0000000001100000 | 96 | for X2 |
| memory[15] | 0000000000000001 | 1 | sentence number |
| memory[16] | 0000000000000001 | 1 | word number |

- Click Button [LoadProgram2]

That will store instructions used program2 into memory [64-100] and set the PC =64.

For more details of programs, please check program2.xlsx

If you want to load program2 manually, make sure program2 is stored from memory [64].

- Click Button [ReadFile]

Our simulator will read “program2_paragraph.txt” and store each word into memory automatically.

(“simulator-1.0-SNAPSHOT.jar” and “program2_paragraph.txt” should be in the same folder.)

- Input a word here, such as “system,” then click [InputAWord].

- Then click [Run] or [Step] to run the program2. After executing instructions orderly, the Simulator Log will output information about what our simulator has done.

- The word, sentence number and word number will output in Simulator Console.

A quick test on Instructions (included in Part2)

We have pre-stored a “Load and Store Test Program” supporting quick experiments.

- Select [Programls], it will be loaded automatically.

- Input a number in the TextField if you need it.

- Click Button [Load], and then the “programls” will be loaded to memory start from StartIndex(input in last step) or Default Location(no input), which is 32.

- Then click [Run] or [Step] to run it.

Test instructions freely (included in Part2)

You can use instructions freely by choosing [FreeRun] and set your own instructions.

- Input an instruction in Program Editor.

Such as “0010000000011111” (JZ R0,0,31)

- Input in TextField an memory index where the instruction will be sored. E.g., we store it in memory[20]

- Click [Run] or [Step], this instruction is executed

Tests for Machine Fault and Trap (included in Part3)

- Illegal Memory Address to Reserved Locations MFR set to binary 0001

- Illegal TRAP code  MFR set to binary 0010

- Illegal Operation Code MFR set to 0100

- Illegal Memory Address beyond 2048 (memory installed)  MFR set to binary 1000

- Illegal Memory Address to Reserved Locations

STR R0,0,2 0000100000000010

Memory[2] is a reserved location, so it will throw IllegalMemoryAccess Exception.

- Illegal TRAP code

0111100000000000 TRAP 0

We don’t have routine 0, so Trap 0 will throw IllegalTrapCode Exception.

- Illegal Operation Code

1111110000000000

“111111” is not an operation type, so it will throw IllegalOpcode Exception.

- Illegal Memory Address beyond 2048

Memory [21] = 2049

STR R0,1,21 0000100000110101

Memory[2049] is out of bounds, so it will throw MemoryOutOfBounds Exception.

- Trap

0111100000000001 Trap 1: IllegalMemoryAccess

0111100000000010 Trap 2: IllegalTrapCode

0111100000000011 Trap 3: IllegalValue

0111100000000100 Trap 4: IllegalOpcode

0111100000000101Trap 5: IllegalRegisterAccess

0111100000001000 Trap 8: MemoryOutOfBounds